034054 Mechanical Design 1M

Homework4: Turntable Drive System Design

Tao Wang (999020910)

Lining Xie (999021090)

Dongming Zheng (999021371)

Zhuorui Yun (999021264)

Li Yan (999021157)

Abstract

This report serves as an extension of the design project initiated in Homework 3, which involved the design of a mechanical turntable to support a heavy payload of 9.5 × 10³ kg in rotation. The previous assignment focused on the structural integrity and bearing selection for the turntable itself. In this assignment (Homework 4), the objective shifts to the design of the drive system necessary for the efficient and safe operation of the turntable. The main tasks of this report include the selection and specification of a suitable standard (off-the-shelf) reduction gearbox, as well as determining the required rated power and nominal speed for a three-phase electric motor. The design takes into account a variable-speed control system capable of adjusting the motor speed by a factor η (0.6 ≤ η ≤ 1.5) around the nominal operating speed. Accordingly, the gearbox selection process emphasizes reliability and the capability to meet torque requirements across this specified speed range.

New Design

Our new design incorporates the addition of a double-layer track beneath the structure’s original rollers, with the track securely fastened using M10 bolts. This configuration distributes loads to mitigate deformation risks; meanwhile, the double-layer structure delivers dual support, minimizing vibration and positional deviation during movement—thereby ensuring smoother travel of the circular assembly. It also allows for more precise control over the movement path and prevents misalignment, rendering it ideal for fixed-route material handling applications. Below are the detailed design drawings and cross-sectional views of this setup.

Figure1. Design main body

Figure2. Cross-section of the track

Figure3. Engineering drawing of the main part

Material Selection and Properties

For this design, I selected AISI 1045 steel as the material for the shaft. According to the material tables, the mechanical properties are:

- Ultimate Tensile Strength: Sut=625 MPa

- Yield Strength: Sy=530 MPa

- Elastic Modulus: E=2.05×1011 Pa

First, I estimated the uncorrected endurance limit (Se'). Since the tensile strength is below 1400 MPa, we can use the standard approximation:

Se'=0.5Sut

Se'=0.5×625 MPa=312.5 MPa

Marin Factors

To find the actual endurance limit (Se) for the real part, I applied the Marin modification factors as taught in class. The formula is:

Se=kakbkckeSe'

Here is how I determined each factor:

Surface Factor (ka):

The part will be machined. Using the parameters a=3.04 and b=-0.217 from the textbook:

ka=aSutb=3.04625-0.217=0.7519

Size Factor (kb):

Since the diameter is relatively small, the size effect is not significant, so I assumed:

kb=1

Load Factor (kc):

The shaft experiences combined loading. For conservative design regarding torsion/shear, I used:

kc=0.59

Reliability Factor (ke):

I want a high reliability of 99.9% for this component. The corresponding factor is:

ke=0.753

Resulting Endurance Limit:

Plugging these values back into the equation:

Se=0.7519×1×0.59×0.753×312.5 MPa

Se≈104.39 MPa

Stress Concentration

Since there is a change in the geometry (like a shoulder or groove), I need to calculate the stress concentration factor to see how much the stress increases locally.

The maximum stresses are given by:

σmax=Kfσ0 and τmax=Kfsτ0

To find Kf, I first looked up the theoretical stress concentration factor (Kt) and then calculated the notch sensitivity (q).

Theoretical Factor (Kt):

For the geometry with diameters d=10 and w=20, the ratio is d/w = 0.5. Checking the chart for this ratio, I found:

Kt=2.18

Notch Sensitivity (q) & Neuber Constant (a):

I calculated the Neuber constant a using the equation for steel based on Sut:

a=1.24-2.25×10-3Sut+1.6×10-6Sut2-4.11×10-10Sut3

Substituting Sut=625, I got:

a≈0.3584 mm

Fatigue Stress Concentration Factor (Kf):

I assumed a radius of r=5 mm for the fillet/notch (approximating an M10 thread root or shoulder). The final factor is:

Kf=1+qkt-1 and Kfs=1+qsKts-1

We design that d=10mm and w=20mm, so d/w=10/20=0.5

From the plot we can find that Kt=2.18

a=1.24-2.25*10-3Sut+1.6*10-6Sut2-4.11*10-10Sut3=0.3584

We assume r=5mm (M10) , according to the formula Kf=1+Kt-11+ar=2.017

Fluctuating force:

σmin=0 , σmax=26.15 MPa  rtot=1.47372+1.157922=1.8742

Fr=mrw2=Fr,tot=9.5*103×1.8742×15×2π602=43931.195 N=43.931 kN

For one side orbit: Feach=Fr2=21.965 kN

For each bolt: Fbult=Fr4=4.393 kN

σa=σmax-σmin2=13.075=σm

σa'=σm'=kf*13.075=26.372 MPa

So the safety factor is: nf=σa'Se+σm'Sut-1=26.372104.3896+26.372625-1=3.39

σeq=σa1-σmSut=27.534 MPa

And the finite fatigue: nf=Syσeq=22.88

Motor selection

Floor: Ifloor=38534 kg·m

Moment inertia for three loads:

Iload=IA+IB+IC=3000×13+5000×17+1500×4=130000 kg⋅m2

Total moment of inertia: Itot=130000+38534=168534 kg⋅m2

External gear ratio: 144:20 = 7.2:1

So gear ratio in gear unit is ≈1007.2=13.89

Angular velocity: ω=2π60⋅15=1.57 rad/s

Angular acceleration: α=π16

Torque output = J×α=168534×π16=33091.57 Nm

The torque input by motor should be Tin=Tout100=330.92 Nm

Operation torque from gear unit: 330.92×13.89=4596.1 Nm

Power of motor: P=T×ω≈52Kw

High-Speed Coupling:

The coupling is installed between the 110 kW Electric Motor (output shaft ϕ 65 mm) and the Gear Unit input shaft (ϕ 50  mm). Its primary function is to transmit the motor's torque while compensating for minor angular, parallel, and axial misalignments, and damping the vibrations during the 8-second acceleration phase.

Design Parameters & Requirements

- Nominal Speed (n): 1500 rpm

- Input Shaft Diameter (Din): 65 mm (Motor side).

- Output Shaft Diameter (Dout):50 mm (Gearbox side).

The torque produced by the motor is 300Nm as calculated before. Based on the dynamic requirements of the 9.5-metric ton turntable system, the R+W EK1 4500 A Elastomer Coupling was selected for the high-speed interface between the motor and the gear unit.

The EK1 4500 model is rated for a nominal torque of 5,000 Nm. This selection provide a safety factor of approximately 15x over the nominal motor torque. This high margin is critical for handling the inertial shock loads during the mandatory 8-second acceleration phase of the 9.5-ton payload.

To find the proper bolt, we did the following calculation.

Using moment balance:
P*66=21965.597×152
where
F=21965.597 N

So we get:

P=50587.4355N

Each of them share forced
P2=25293.72 N
So the stress is:

σb=322.05MPa

We take property class 10.9, from the table we know

Sp=830MPa

From this table, we know

At=58mm2

Fi=0.75AtSp

So we calculate:

σb=FbAt=cP+Fi At=0.3×25293.72+3610558=753.33 MPa

Under this circumstance, the safety factor is:

np=Spσb=1.102

It might be too low, so we consider Fi=10000N

Now,

σb=303.243MPa

So np=Spσb=2.737

We consider it a good design.

Next we calculate the fatigue:

Fbmin=104

Fbmax=0.3×25293.72+104=17588.116N

σm =(Fbmax+Fbmin)/2At=(17599.1+104)/258=237.83 MPa

σa =(Fbmax-Fbmin)/2At=65.415 MPa
σi=10458=172.41MPa

Se=162 MPa

Sut=1000 MPa

Finally, we can find that the safety factor is:

nf=Se(Sut-σi)Sut σa+Se(σm-σi)=162(1000-172.41)1000*65.415+162*(237.83-172.41)=1.764

We consider its a good design.

Here is the point-by-point explanation in English for the selection of the motor (SEW Eurodrive K167DRN200L4TF, 30 kW):

Figure4. Driven Selection

Figure5. Reduction drive drawings

Figure6. 3D view of the Reduction drive

1. Power Matching

Initial calculations indicated a theoretical power requirement of approximately 52 kW. However, a detailed selection analysis revealed that the system's actual peak power demand is around 33 kW. The selected 30 kW motor provides good overload capacity (with a maximum utilization of only 86%) and is paired with a recommended 30 kW frequency inverter, fully meeting the demands of dynamic operating conditions.

2. Torque Capability

The motor-gear unit offers a rated output torque of 17,600 Nm and a maximum allowable torque of 35,000 Nm. This significantly exceeds the calculated operational torque requirement of approximately 4,596 Nm, providing a sufficient torque reserve.

3. Speed Matching

With a total gear reduction ratio of 109.83, the motor's rated speed of 1,783 rpm is reduced to an output speed of 16 rpm. This closely matches the target speed of 15 rpm, meeting the turntable's speed requirement.

4. Reliability and Adaptability

The unit features a high service factor of 2, enabling it to reliably withstand load fluctuations caused by the speed adjustment range (η = 0.6–1.5). This makes it suitable for variable-speed operating conditions.

5. Energy Efficiency and Protection

The motor complies with the IE3 efficiency standard, achieving over 94% efficiency, which helps reduce long-term energy consumption. Its IP54 protection rating makes it suitable for typical industrial environments.

6. Compatibility and Control

A 30 kW frequency inverter is recommended for pairing, facilitating motor speed control and meeting the system's requirement for adjustable speed.

In summary, this motor meets the design requirements in terms of power, torque, speed, reliability, energy efficiency, and control compatibility, representing a rational and reliable engineering choice.

Catalogue Source：seweurodrive.com/os/dud?search=K167DRN200L4/TF&tab=documents

CAD Specification：seweurodrive.com/os/dud?search=K167DRN200L4/TF